Требования для возможности подкачки боковой поддержки в режиме спорт

SMFx Requirements

- Front SM shall transmit Side_Acceleration(0x1) via LIN, if all of the following conditions are met:

  - DrivingMode_Sig(0x1) received via Body-CAN;

  - OCUFx_SeatOccupied_Stat(0x1) received via Body-CAN.

  - VehicleSpeed is equal or more than 4 km/h received via Body-CAN.

- Front SM shall transmit Side_Acceleration(0x0) via LIN, if any of the following conditions are met:

  - DrivingMode_Sig(0x0) received via Body-CAN;

  - OCUxx_SeatOccupied_Stat(0x0) received via Body-CAN;

  - VehicleSpeed is less than 4 km/h received via Body-CAN

MLSM requirements

- MLSM shall ignore the following values of signal:

  - Sidesupport_backrest(0x1 OR 0x2) received via LIN;

  - Sidesupport_cushion(0x1 OR 0x2) received via LIN;

  - Massage_on(0x1) received via LIN;

if:

  - Side_Acceleration(0x1) received via LIN;

- MLSM shall set (O_CushionSupRight AND O_CushionSupLeft AND O_Pump) active, if all of the following conditions are met:

  - Sidesupport_on(0x1) received via LIN;

  - Side_Acceleration(0x1) received via LIN;

  - Current value of pressure in the cushion support bags is less than MAX kPa.

- MLSM shall set (O_CushionSupRight AND O_CushionSupLeft AND O_Pump) inactive, if any of the following conditions are met:

  - Current value of pressure in the cushion support bags is more or equal to MAX kPa;

  - Sidesupport_on changed to 0x0 received via LIN;

  - Side_Acceleration changed to 0x0 received via LIN.

- MLSM shall set (O_CushionSupRight AND O_CushionSupLeft AND O_Drain) active, if any of the following conditions are met:

- Sidesupport_on changed to 0x0 received via LIN;

- Side_Acceleration changed to 0x0 received via LIN;

- MLSM shall set (O_CushionSupRight AND O_CushionSupLeft AND O_Drain) inactive, if all of the following conditions are met:

  - Sidesupport_on changed to 0x1 received via LIN;

  - Side_Acceleration changed to 0x1 received via LIN;

  - Current value of pressure in the cushion support bags is equal to 0 kPa.

- MLSM shall set (O_BackrestSupRight AND O_BackrestSupLeft AND O_Pump) active, if all of the following conditions are met:

- Sidesupport_on(0x1) received via LIN;

- Side_Acceleration(0x1) received via LIN;

- Current value of pressure in the cushion support bags is less than MAX kPa.

- MLSM shall set (O_BackrestSupRight AND O_BackrestSupLeft AND O_Pump) inactive, if any of the following conditions are met:

  - Current value of pressure in the cushion support bags is more or equal to MAX kPa;

  - Sidesupport_on changed to 0x0 received via LIN;

  - Side_Acceleration changed to 0x0 received via LIN.

- MLSM shall set (O_BackrestSupRight AND O_BackrestSupLeft AND O_Drain) active, if any of the following conditions are met:

- Sidesupport_on changed to 0x0 received via LIN;

- Side_Acceleration changed to 0x0 received via LIN;

- MLSM shall set (O_BackrestSupRight AND O_BackrestSupLeft AND O_Drain) inactive, if any of the following conditions are met:

  - Sidesupport_on changed to 0x1 received via LIN;

  - Side_Acceleration changed to 0x1 received via LIN;

  - Current value of pressure in the cushion support bags is equal to 0, kPa.